EXPERIMENT : FLAME SENSOR Alert System Using DE0- Nano FPGA with Buzzer and LED Indicators

Objective:

To design and implement a flame detection system using Verilog HDL on the DE0-Nano FPGA board.

The system reads input from a flame sensor, and in case of flame detection, triggers a buzzer and LED indicators.

This project demonstrates real-time hazard detection using digital control logic and sensor interfacing.

Materials Required:

- DE0-Nano FPGA Development Board

- Flame Sensor (Digital Output)

- Active Buzzer Module

-  Jumper Wires

- Intel Quartus Prime Lite Edition

- USB Blaster (for programming FPGA)

- PC or Laptop

- CP2102 USB-to-Serial Converter (optional, for UART debugging)

Theory:

The flame sensor is an infrared sensor module that detects fire or IR light from flames. It gives a digital LOW (0) signal when a flame is detected, and HIGH (1) when not.

In this project:

- If the sensor detects flame → Buzzer ON + Both LEDs blink

- If no flame is present → Only one LED blinks, and buzzer remains OFF

The logic is implemented using Verilog and synthesized on the DE0-Nano FPGA board.

Logic Expressions / Functionality:

| Sensor Output | LED Output | Buzzer |
| --- | --- | --- |
| 1 (No flame) | led_output= 2'b01 (LED0 ON) | OFF |
| 0 (Flame detected) | led_output= 2'b11 (LED0 & LED1 ON) | ON |

Experimental Procedure:

Hardware Connections:

Flame Sensor VCC   → JP1 pin 11 (3.3V)

Flame Sensor GND   → JP1 pin 12 (GND)

Flame Sensor OUT   → JP1 pin 2 (GPIO_0[0])

Buzzer VCC         → JP2 pin 2 (GPIO_1[0])

Buzzer GND         → JP2 pin 12 (GND)

FPGA Pin Assignments:

| Signal Name | Direction | Pin | Description |
| --- | --- | --- | --- |
| flame_do | Input | PIN_D3 | Flame sensor digital output |
| clk | Input | PIN_R8 | Clock signal |
| buzzer | Output | PIN_F13 | Controls buzzer |
| led_output[0] | Output | PIN_A15 | LED0 indicator |
| led_output[1] | Output | PIN_A13 | LED1 indicator |

Pin Planner Screenshot:

The image below shows the pin assignments done in Intel Quartus Prime using the Pin Planner tool for the flame sensor, buzzer, and LEDs.

Figure 1: Screenshot of Pin Planner in Quartus showing assigned FPGA pins.

Verilog Code :

Main Module :

module flame_sensor ( input clk, input flame_d0, // Input from flame sensor D0 output reg [1:0] leds, // Output to two LEDs output reg buzzer_out // Output to buzzer ); reg [23:0] counter = 0; // For blinking delay reg blink = 0; always @(posedge clk) begin // Simple counter for blinking (~half-second blink on 50 MHz clock) counter <= counter + 1; if (counter == 24'd5000000) begin blink <= ~blink; counter <= 0; end if (flame_d0 == 0) begin // Flame Detected leds[0] <= blink; leds[1] <= blink; buzzer_out <= 1'b1; // Buzzer ON end else begin // No Flame leds[0] <= blink; leds[1] <= 1'b0; buzzer_out <= 1'b0; // Buzzer OFF end end endmodule

Top Module:

module top_flame_sensor ( input clk, // 50 MHz system clock input flame_d0, // From flame sensor D0 (GPIO_00) output [1:0] led_output, // Connects to LEDR[1:0] output buzzer // Connect this to GPIO pin for buzzer ); flame_sensor u_flame_sensor ( .clk(clk), .flame_d0(flame_d0), .leds(led_output), .buzzer_out(buzzer) ); endmodule

Project Setup Photo:

The following image shows the actual hardware setup of the flame sensor with the DE0-Nano FPGA board and buzzer connected as per the described connections.

Figure 2: Working prototype showing flame sensor, LEDs, and buzzer on DE0-Nano.

Expected Output:

- No Flame:

LED0 glows

Buzzer OFF

- Flame Detected:

LED0 and LED1 both glow

Buzzer ON

Applications:

- Fire and flame detection in embedded systems

- Industrial safety and monitoring

- Early warning systems in hazardous environments

- Educational labs for real-time sensor interfacing

Conclusion:

The flame detection system was successfully implemented using Verilog and tested on the DE0-Nano FPGA board.

It effectively detects flame presence and triggers visual (LED) and auditory (buzzer) alerts.

This project provided practical experience in interfacing sensors with FPGA and designing real-time digital logic systems.